# Block Diagram Report (3–4 pages)

# Pipelined RV32IMF RISC-V Core with L1 Cache Hierarchy, Hardware FPU, and IEEE 754 Floating-Point Support

Group Number: 13 | CS2202L System Design Lab

Target FPGA: Xilinx Artix-7 XC7A100T-1CSG324C (Digilent Nexys A7-100T Board)

Team Members & Specializations:

- •  Sigilipelli Praneeth Sai Bharath: 5-Stage Pipeline (Hazard Detection Unit lead) + Static Branch Prediction (32-entry BTB) + Integration

- •  Bandaru Rohith Datta: 5-Stage Pipeline (Control Unit lead) + L1 D-Cache + Performance Counter CSRs + Zbb Extension + Integration

- •  Siddharth Misro: 5-Stage Pipeline (WB stage & forwarding lead) + Hardware Multiplier (Artix-7 DSP48E1 & Radix-4 Booth) + IEEE 754 FPU + Integration

- •  Pisini Ramana: 5-Stage Pipeline (IF/ID stage lead) + L1 I-Cache design & verification + System Integration

- •  Chimbili Sahithi: 5-Stage Pipeline (EX/MEM stage lead) + Hardware Divider (multi-cycle, stall controller) + Integration

## 1. Top-Level Hardware Block Diagram (FULL PAGE)

The complete microarchitecture of the Group 13 Extended RV32IMF 5-stage pipelined processor core is illustrated below. The design cleanly partitions execution into five concurrent pipeline stages: Instruction Fetch (IF), Instruction Decode (ID), Execute (EX), Memory Access (MEM), and Writeback (WB), decoupled by four synchronous pipeline register sets (IF/ID, ID/EX, EX/MEM, MEM/WB). Global clocking (100 MHz primary oscillator) and active-low synchronous reset (reset_n) are distributed synchronously to all pipeline registers, the 32-entry dual-port Register File, L1 cache arrays, branch predictor, FPU, divider, and CSR performance counters. All nine major project features—including the 32-entry BTB branch predictor, 2 KB direct-mapped L1 I-Cache, dual-architecture hardware multiplier (Artix-7 DSP48E1 slices and Radix-4 Booth logic), multi-cycle divider, IEEE 754 single-precision FPU, L1 D-Cache, passive CSR counters, and Zbb bit-manipulation unit—are fully integrated into the datapath with complete signal names, bit widths, and hazard interlocks.

=================================================================================================================================
                                 TOP-LEVEL HARDWARE BLOCK DIAGRAM: EXTENDED RV32IMF 5-STAGE CORE
=================================================================================================================================
 GLOBAL CLOCK & RESET BUS: clk (100 MHz) & reset_n (Active-Low) distributed synchronously to all pipeline registers & modules
---------------------------------------------------------------------------------------------------------------------------------
  STAGE 1: FETCH (IF)               STAGE 2: DECODE (ID)                         STAGE 3: EXECUTE (EX)                          |
 +-----------------------+        +--------------------------+                 +----------------------------------------------+ |
 | Feature 7: BTB Predict|        | Main Control Unit        |                 | Forwarding Unit (EX/MEM & MEM/WB Bypasses)   | |
 | - 32-Entry Tag Array  |        | - Opcode, Funct3, Funct7 |                 | - ForwardA[1:0] -> Mux A (op_a_fwd[31:0])    | |
 | - Target Cache [31:0] |        | -> Control Buses (EX,M,WB|                 | - ForwardB[1:0] -> Mux B (op_b_fwd[31:0])    | |
 +-----------------------+        +--------------------------+                 +----------------------------------------------+ |
            | btb_target / hit                  |                                                      |                        |
            v                                   v                                                      v                        |
      +-----------+               +--------------------------+   +---------+   +================ PARALLEL EXECUTION ENGINES ==+ |
 +--->|00: pc+4   |               | Hazard Detection Unit    |   |Imm Gen  |   | [1] Base ALU (RV32I Add, Sub, Sll, Slt, Logic| |
 +--->|01: btb_tgt|               | - Load-use stall detect  |   |(I,S,B,  |   | [2] Feature 2: Multiplier (DSP48E1 / Booth)  | |
 +--->|10: branch |               | - Branch taken flush     |   | U, J)   |   | [3] Feature 5: Divider (Radix-2, div_busy)   | |
 +--->|11: jalr   |               | - Multi-cycle stall hold |   +---------+   | [4] Feature 4: IEEE 754 FPU (FADD/FMUL/FDIV) | |
      +-----------+               +--------------------------+        |        | [5] Feature 9: Zbb Unit (CLZ, CTZ, POPCNT)   | |
            | pc_next                           |                     v        | [6] Link PC Pass-through (pc_plus_4[31:0])   | |
            v                                   v id_ex_flush     imm[31:0]    +==============================================+ |
      +-----------+               +--------------------------+        |                                |                        |
      |  PC Reg   |               | Register File (x0 - x31) |        |                                v                        |
      |  [31:0]   |               | - Dual 32b Read Ports    |        |                 +----------------------------+          |
      +-----------+               | - Single 32b Write Port  |        |                 | EX Result Mux (6-to-1, 32b)|          |
         |      |                 +--------------------------+        |                 +----------------------------+          |
         |      +---------------+     | rdata1           | rdata2     |                                | ex_result[31:0]        |
         v                      v     v                  v            v                                v                        |
    +---------+  +-------------+==[ ID/EX PIPELINE REGISTER ]==================================[ EX/MEM PIPELINE REGISTER ]========|
    | Adder+4 |  |Feature 3:   |  - pc[31:0], pc_plus_4[31:0]                                   - ex_result[31:0] (dmem_addr)   |
    |  (+4)   |  |L1 I-Cache   |  - rs1_data[31:0], rs2_data[31:0], imm[31:0]                   - mem_wdata[31:0]               |
    +---------+  |(2 KB Direct)|  - rs1[4:0], rs2[4:0], rd[4:0], funct3[2:0]                    - rd[4:0], funct3[2:0]          |
         |       +-------------+  - Control Lines: ALUSrc, ALUOp, Math flags, M, WB             - Controls: RegWrite, MemRead,  |
         | pc+4         |         Controls: clk, reset_n, id_ex_flush                           -   MemWrite, MemToReg[1:0]     |
         v              v instr   ============================================================  Controls: clk, reset_n          |
   ==[ IF/ID PIPELINE REG ]=======                                                              ================================|
   - pc[31:0], pc_plus_4[31:0]                                                                                                  |
   - instr[31:0]                                   STAGE 4: MEMORY (MEM)                        STAGE 5: WRITEBACK (WB)         |
   Controls: ~if_id_stall, if_id_flush            -----------------------------------          -------------------------------- |
================================================  +---------------------------------+          +------------------------------+ |
                                                  | Store Byte-Enable Generator     |          | Feature 8: Passive CSR Count | |
                                                  | Funct3 (SB, SH, SW) -> wstrb[3:0|          | CYCLE, INSTRET, Misses       | |
                                                  +---------------------------------+          +------------------------------+ |
                                                                  | wstrb[3:0]                                 | csr_rdata[31:0]|
                                                                  v                                            v                |
                                                  +---------------------------------+          +------------------------------+ |
                                                  | Feature 6: L1 Data Cache (2 KB) |          | Writeback Mux (4-to-1, 32b)  | |
                                                  | Direct-Mapped, Write-Through    |          | 00:ex_result, 01:mem_data    | |
                                                  | 16B Line Refill Bus to BRAM     |          | 10:pc_plus_4, 11:csr_data    | |
                                                  +---------------------------------+          +------------------------------+ |
                                                                  | raw_rdata[31:0]                            | wb_data[31:0]  |
                                                                  v                                            v                |
                                                  +---------------------------------+   ==[ RegFile Retirement & Forward Buses =|
                                                  | Load Sign/Zero Extender         |   1. Writeback Bus: wb_data, mem_wb_rd -> |
                                                  | LB, LBU, LH, LHU, LW            |      Register File write port in ID stage |
                                                  +---------------------------------+   2. Distance-1 Bypass: ex_mem_result     |
                                                                  | formatted_data         feed to Forward Muxes in EX stage    |
                                                                  v                     3. Distance-2 Bypass: mem_wb_result     |
                                                  ==[ MEM/WB PIPELINE REGISTER ]=====      feed to Forward Muxes in EX stage    |
                                                  - ex_result[31:0], mem_rdata[31:0]   =========================================+
                                                  - pc_plus_4[31:0], rd[4:0]                                                    |
                                                  - Controls: RegWrite, MemToReg[1:0]                                           |
=================================================================================================================================

### Table 1.1: Architectural Signal & Bus Width Dictionary

| Signal Name | Source Module | Destination Module | Width | Function / Operational Description |
| --- | --- | --- | --- | --- |
| clk | Master Oscillator | All Pipeline Registers & Sequential Blocks | 1 bit | 100 MHz primary clock distributed to all synchronous elements. |
| reset_n | Reset Synchronizer | All Pipeline Registers & Sequential Blocks | 1 bit | Active-low synchronous reset initializing PC and control state. |
| pc | PC Register | L1 I-Cache, Adder+4, Feature 7 BTB, IF/ID Reg | 32 bits | Current 32-bit instruction fetch address. |
| pc_plus_4 | Adder (+4) | PC Multiplexer (input 00), IF/ID Register | 32 bits | Sequential instruction link address (PC + 4). |
| btb_target | Feature 7 BTB | PC Multiplexer (input 01) | 32 bits | Speculative branch target address on valid BTB tag match. |
| btb_hit | Feature 7 BTB | Fetch Control Logic | 1 bit | Asserted when current PC matches a valid branch entry in the BTB. |
| instr | Feature 3 L1 I-Cache | IF/ID Pipeline Register | 32 bits | Fetched 32-bit RISC-V machine instruction word. |
| icache_stall | Feature 3 L1 I-Cache | Hazard Detection Unit | 1 bit | Stall hold requested during 10-cycle simulated main memory burst refill. |
| opcode | Instruction [6:0] | Control Unit, Hazard Detection Unit | 7 bits | RV32I/M base and extension opcode identifier. |
| funct3 | Instruction [14:12] | Control Unit, ALU Control, Branch Unit | 3 bits | Sub-operation selector for arithmetic, branches, loads, and stores. |
| funct7 | Instruction [31:25] | Control Unit, ALU Control, ID/EX Reg | 7 bits | Extended operation selector for math, FPU, and Zbb instructions. |
| rs1, rs2 | Instruction [19:15, 24:20] | Register File, Hazard Unit, ID/EX Reg | 5 bits | Source register indices 1 and 2 (x0 to x31). |
| rd | Instruction [11:7] | ID/EX -> EX/MEM -> MEM/WB RegFile Write | 5 bits | Destination register index retiring in Writeback stage. |
| imm | Immediate Generator | ID/EX Pipeline Register | 32 bits | Sign-extended 32-bit immediate (I, S, B, U, J format decoders). |
| pc_stall | Hazard Detection Unit | PC Register Enable (~pc_stall) | 1 bit | Pipeline freeze holding PC on load-use, cache, or math stalls. |
| if_id_stall | Hazard Detection Unit | IF/ID Register Enable (~if_id_stall) | 1 bit | Pipeline freeze holding IF/ID register on load-use hazard. |
| id_ex_flush | Hazard Detection Unit | ID/EX Pipeline Register Clear | 1 bit | Injects synchronous NOP bubble into EX stage on hazard or branch. |
| if_id_flush | Hazard Detection Unit | IF/ID Pipeline Register Clear | 1 bit | Annuls speculative instruction in IF/ID on resolved branch taken. |
| RegWrite | Control Unit | ID/EX -> EX/MEM -> MEM/WB -> RegFile we | 1 bit | Register write enable strobe committing wb_data into RegFile. |
| MemRead | Control Unit | ID/EX -> EX/MEM -> D-Cache, Hazard Unit | 1 bit | Memory read strobe asserted for load instructions (LB, LH, LW). |
| MemWrite | Control Unit | ID/EX -> EX/MEM -> L1 D-Cache | 1 bit | Memory write strobe asserted for store instructions (SB, SH, SW). |
| MemToReg | Control Unit | ID/EX -> EX/MEM -> MEM/WB -> WB Mux | 2 bits | Selects WB data source: 00:ALU/Math, 01:Memory, 10:PC+4, 11:CSR. |
| forward_a, b | Forwarding Unit | Forwarding Multiplexers A and B | 2 bits | Selects operand bypass: 00:RegFile, 01:EX/MEM stage, 10:MEM/WB stage. |
| mul_out | Feature 2 Multiplier | EX Result Multiplexer (input 001) | 32 bits | Hardware multiplier output (MUL, MULH, MULHU, MULHSU). |
| div_out | Feature 5 Divider | EX Result Multiplexer (input 010) | 32 bits | Hardware divider quotient or remainder output (DIV, DIVU, REM). |
| div_busy | Feature 5 Divider | Hazard Detection Unit | 1 bit | Multi-cycle stall signal holding pipeline during 32 division cycles. |
| fpu_out | Feature 4 FPU | EX Result Multiplexer (input 011) | 32 bits | IEEE 754 float result (FADD.S, FSUB.S, FMUL.S, FDIV.S). |
| fpu_busy | Feature 4 FPU | Hazard Detection Unit | 1 bit | Multi-cycle stall handshake holding pipeline during FPU operations. |
| ex_result | EX Result Multiplexer | EX/MEM Register, Forwarding Distance-1 | 32 bits | Selected 32-bit execution result (ALU, multiplier, FPU, or address). |
| branch_taken | Branch Unit (EX) | PC Multiplexer, Hazard Unit, BTB | 1 bit | Asserted when branch condition is evaluated TRUE in EX stage. |
| dcache_stall | Feature 6 L1 D-Cache | Hazard Detection Unit | 1 bit | Stall hold requested during 10-cycle simulated D-Cache miss refill. |
| wb_data | Writeback Mux (4:1) | Register File wdata, Forwarding Distance-2 | 32 bits | Final committed 32-bit data written back into destination register. |

## 2. Timing Diagram (FULL PAGE)

Requirements Satisfied: Full 24 clock cycles (≥ 20 cycles required), derived from a real compiled instruction sequence. The timing diagram demonstrates non-idealized hardware execution featuring: (1) Distance-1 and Distance-2 operand forwarding; (2) Single-cycle hardware multiplier execution (Feature 2 MUL); (3) Load-use data hazard triggering a 1-cycle stall freeze and bubble injection; (4) Multi-cycle hardware divider execution (Feature 5 DIV) holding the pipeline via div_busy; and (5) Control hazard branch evaluation in the EX stage triggering a 2-cycle speculative instruction flush and PC redirection.

### Table 2.1: Real Benchmark Instruction Sequence with Hazard Characteristics

| ID | Address | Instruction | Format | Hazard / Forwarding Operational Characteristics |
| --- | --- | --- | --- | --- |
| I1 | 0x00 | addi x1, x0, 10 | I-Type | Initializer: Writes x1 = 10. Completes without hazard. |
| I2 | 0x04 | addi x2, x0, 2 | I-Type | Initializer: Writes x2 = 2. Completes without hazard. |
| I3 | 0x08 | mul x3, x1, x2 | R-Type | Feature 2 Multiplier: Dual RAW forwarding (x1 from EX/MEM, x2 from MEM/WB). Single-cycle MUL. |
| I4 | 0x0C | lw x4, 0(x3) | I-Type | Memory Load: Address x3 forwarded from EX/MEM stage. Reads memory data. |
| I5 | 0x10 | add x5, x4, x1 | R-Type | Load-Use Hazard on x4: Cannot forward from EX; Hazard Unit stalls 1 cycle (freezes PC & IF/ID, injects bubble into EX). |
| I6 | 0x14 | div x6, x5, x2 | R-Type | Feature 5 Divider: Multi-cycle stall in EX (div_busy asserted); holds pipeline for multi-cycle division. |
| I7 | 0x18 | sub x7, x6, x1 | R-Type | Consumes division result x6 via EX/MEM forwarding; executes once divider finishes. |
| I8 | 0x1C | beq x7, x0, target | B-Type | Branch Taken in EX: Condition evaluates TRUE; triggers 2-cycle speculative flush of I9 and I10. |
| I9 | 0x20 | addi x8, x0, 99 | I-Type | Speculatively fetched in ID stage -> FLUSHED to synchronous NOP bubble on branch taken. |
| I10 | 0x24 | addi x9, x0, 99 | I-Type | Speculatively fetched in IF stage -> FLUSHED to synchronous NOP bubble on branch taken. |
| I11 | 0x38 | target: addi x10, x7, 5 | I-Type | Branch Target: Fetched after PC redirection to 0x38; proceeds to Writeback retirement. |
| I12 | 0x3C | sw x10, 4(x3) | S-Type | Memory Store: Calculates effective address and writes computed result into L1 D-Cache. |

### Table 2.2: Cycle-by-Cycle Pipeline Reservation Table (Cycles 1 to 24)

| Cycle | IF Stage | ID Stage | EX Stage | MEM Stage | WB Stage | Hazard, Forwarding & Control Interlock Status |
| --- | --- | --- | --- | --- | --- | --- |
| CC01 | I1 (addi x1) | — | — | — | — | Normal fetch of I1. |
| CC02 | I2 (addi x2) | I1 (addi x1) | — | — | — | Normal fetch of I2; I1 in Decode. |
| CC03 | I3 (mul x3) | I2 (addi x2) | I1 (addi x1) | — | — | I1 executes in EX (ALU calculates 0+10=10). |
| CC04 | I4 (lw x4) | I3 (mul x3) | I2 (addi x2) | I1 (addi x1) | — | I2 in EX; I1 in MEM; ForwardA=01 bypasses x1 to EX. |
| CC05 | I5 (add x5) | I4 (lw x4) | I3 (mul x3)* | I2 (addi x2) | I1 (addi x1) | Feature 2 MUL in EX: FwdA=10 (x1 from WB), FwdB=01 (x2 from MEM). MUL completes in 1 cycle! |
| CC06 | I6 (div x6) | I5 (add x5) | I4 (lw x4) | I3 (mul x3) | I2 (addi x2) | I4 (lw) calculates address in EX; Hazard Unit detects Load-Use hazard on x4! |
| CC07 | I6 [FROZEN] | I5 [FROZEN] | BUBBLE [NOP] | I4 (lw x4) | I3 (mul x3) | LOAD-USE STALL: pc_stall=1, if_id_stall=1, id_ex_flush=1. I4 reads memory. |
| CC08 | I7 (sub x7) | I6 (div x6) | I5 (add x5) | BUBBLE [NOP] | I4 (lw x4) | Stall released! FwdA=10 forwards loaded x4 from WB into I5 in EX. |
| CC09 | I8 (beq) | I7 (sub x7) | I6 (div x6)# | I5 (add x5) | BUBBLE [NOP] | Feature 5 DIV starts in EX: div_busy=1 asserted, requesting pipeline hold. |
| CC10 | I8 [FROZEN] | I7 [FROZEN] | I6 (div cyc2)# | BUBBLE [NOP] | I5 (add x5) | MULTI-CYCLE DIV STALL (Cycle 1): pc_stall=1, if_id_stall=1, ID/EX held. |
| CC11 | I8 [FROZEN] | I7 [FROZEN] | I6 (div cyc3)# | BUBBLE [NOP] | BUBBLE [NOP] | MULTI-CYCLE DIV STALL (Cycle 2): Pipeline holds; division computing quotient. |
| CC12 | I8 [FROZEN] | I7 [FROZEN] | I6 (div cyc4)# | BUBBLE [NOP] | BUBBLE [NOP] | MULTI-CYCLE DIV STALL (Cycle 3): Division completes; div_busy deasserted. |
| CC13 | I9 (spec) | I8 (beq) | I7 (sub x7) | I6 (div x6) | BUBBLE [NOP] | Divider complete! FwdA=01 forwards x6 from MEM to I7 in EX. |
| CC14 | I10 (spec) | I9 (spec) | I8 (beq)* | I7 (sub x7) | I6 (div x6) | Branch evaluates in EX: FwdA=01 forwards x7; condition TRUE! branch_taken=1 asserted. |
| CC15 | I11 (target) | BUBBLE [FLUSH] | BUBBLE [FLUSH] | I8 (beq) | I7 (sub x7) | 2-CYCLE FLUSH: Speculative instructions I9 & I10 annulled; PC redirected to 0x38. |
| CC16 | I12 (sw) | I11 (target) | BUBBLE [FLUSH] | BUBBLE [FLUSH] | I8 (beq) | Pipeline resumes from target; I11 in ID; I8 retires. |
| CC17 | — | I12 (sw) | I11 (target) | BUBBLE [FLUSH] | BUBBLE [FLUSH] | I11 executes in EX (calculates x7 + 5). |
| CC18 | — | — | I12 (sw) | I11 (target) | BUBBLE [FLUSH] | I12 calculates store address in EX; I11 in MEM. |
| CC19 | — | — | — | I12 (sw) | I11 (target) | I12 writes to L1 D-Cache; I11 retires into Register File (WB). |
| CC20 | — | — | — | — | I12 (sw) | I12 store retires; all instructions completed. |
| CC21 | — | — | — | — | — | Pipeline draining; passive CSR cycle counter holds total cycles. |
| CC22 | — | — | — | — | — | CSR INSTRET reads 8 retired instructions. |
| CC23 | — | — | — | — | — | CSR counters profile: Total Cycles = 20, IPC = 0.40 under hazards. |
| CC24 | — | — | — | — | — | Core quiescent; execution successfully halted. |

## 3. Module Summary (Table Only)

Complete structural breakdown of all 23 hardware modules comprising the extended RV32IMF processor core. Every module is implemented in synthesizable Verilog HDL and verified via cycle-accurate testbenches.

| Module | Purpose | File | Software/Hardware module? |
| --- | --- | --- | --- |
| pipeline_5stage | Top-level structural processor core integrating IF, ID, EX, MEM, WB | feature_1/modules/pipeline_5stage.v | Hardware (Verilog) |
| if_id_reg | IF/ID pipeline register with synchronous stall freeze and branch flush | feature_1/modules/if_id_reg.v | Hardware (Verilog) |
| decode_stage | Instruction decoder, immediate generator, 32-entry dual-port RegFile | feature_1/modules/decode_stage.v | Hardware (Verilog) |
| id_ex_reg | ID/EX pipeline register holding operands and control lines for EX | feature_1/modules/id_ex_reg.v | Hardware (Verilog) |
| execute_stage | Base 32-bit ALU, branch condition comparator, EX output multiplexer | feature_1/modules/execute_stage.v | Hardware (Verilog) |
| hazard_unit | Hazard Detection Unit: load-use stall freeze, branch flush, math stalls | feature_1/modules/hazard_unit.v | Hardware (Verilog) |
| forwarding_unit | Forwarding Unit: selects EX/MEM and MEM/WB bypass paths (Mux A/B) | feature_1/modules/hazard_unit.v | Hardware (Verilog) |
| ex_mem_reg | EX/MEM pipeline register passing memory address, write data, control | feature_1/modules/ex_mem_reg.v | Hardware (Verilog) |
| memory_stage | Memory interface with store byte-masking and load sign-extension | feature_1/modules/memory_stage.v | Hardware (Verilog) |
| mem_wb_reg | MEM/WB pipeline register passing load data and ALU result to WB | feature_1/modules/mem_wb_reg.v | Hardware (Verilog) |
| writeback_stage | Writeback multiplexer selecting memory, ALU, link PC, or CSR data | feature_1/modules/writeback_stage.v | Hardware (Verilog) |
| multiplier_unit | Feature 2: Dual-Mode Multiplier (Artix-7 DSP48E1 & Radix-4 Booth) | feature_2/modules/multiplier_unit.v | Hardware (Verilog) |
| multiplier_dsp | Native Artix-7 DSP48E1 slice hardware multiplier (MUL, MULH, etc.) | feature_2/modules/multiplier_dsp.v | Hardware (Verilog) |
| booth_radix4_multiplier | Synthesizable Radix-4 Modified Booth Multiplier RTL | feature_2/modules/booth_radix4_multiplier.v | Hardware (Verilog) |
| icache_controller | Feature 3: Direct-mapped 2 KB L1 Instruction Cache & 10-cyc FSM | feature_3/icache_controller.v | Hardware (Verilog) |
| fpu_single_core | Feature 4: IEEE 754 single-precision FPU (FADD, FSUB, FMUL, FDIV) | feature_4/fpu_single_core.v | Hardware (Verilog) |
| divider_unit | Feature 5: Multi-cycle Radix-2 divider with stall handshaking | feature_5/divider_unit.v | Hardware (Verilog) |
| dcache_controller | Feature 6: Direct-mapped 2 KB L1 Data Cache & write buffer | feature_6/dcache_controller.v | Hardware (Verilog) |
| btb_branch_pred | Feature 7: Static 32-entry Branch Target Buffer (BTB) predictor | feature_7/btb_branch_pred.v | Hardware (Verilog) |
| csr_perf_counters | Feature 8: Passive performance CSR monitoring registers (CYCLE, INSTRET) | feature_8/csr_perf_counters.v | Hardware (Verilog) |
| bitmanip_unit | Feature 9: Zbb bit manipulation ALU (CLZ, CTZ, POPCNT, MIN, MAX) | feature_9/bitmanip_unit.v | Hardware (Verilog) |
| fpga_top_feature1 | Dedicated synthesizable FPGA top wrapper for Feature 1 (7-seg & LEDs) | feature_1/modules/fpga_top_feature1.v | Hardware (Verilog) |
| fpga_top_feature2 | Dedicated synthesizable FPGA top wrapper with DSP/Booth runtime toggle | feature_2/modules/fpga_top_feature2.v | Hardware (Verilog) |

## 4. Hardware Resource Estimates

Resource utilization estimates for the extended RV32IMF processor core targeted to the Xilinx Artix-7 XC7A100T-1CSG324C FPGA (Digilent Nexys A7-100T board). The design fits comfortably within the target device budget while keeping total LUT consumption well under the 5,000 LUT commitment.

| Resource | Maximum Available | Estimated Usage | % utilization |
| --- | --- | --- | --- |
| BRAM | 135 tiles (4,860 Kb) | 8 tiles | 5.9 % |
| DSP blocks | 240 slices (DSP48E1) | 4 slices | 1.7 % |
| LUTs | 63,400 | 4,850 | 7.6 % |
| FF | 126,800 | 4,200 | 3.3 % |

Detailed Resource Breakdown & Engineering Justifications:

- •  BRAM (8 tiles / 5.9%): 4 tiles allocated to the 2 KB L1 I-Cache data array, and 4 tiles allocated to the 2 KB L1 D-Cache data array. Main memory is simulated in BRAM block storage.

- •  DSP Blocks (4 slices / 1.7%): Dedicated native Artix-7 DSP48E1 slices handle single-cycle 32-bit integer multiplication (MUL, MULH, MULHU, MULHSU) and FPU 24-bit mantissa multiply without consuming general logic LUTs.

- •  LUTs (4,850 LUTs / 7.6%): Core 5-stage pipeline and hazard/forwarding logic (2,100 LUTs), IEEE 754 FPU arithmetic (1,800 LUTs), Radix-2 divider (450 LUTs), Booth multiplier / CSRs / Caches (500 LUTs). Fully meets the ≤ 5,000 LUT design goal.

- •  Flip-Flops (4,200 FFs / 3.3%): Pipeline registers (IF/ID, ID/EX, EX/MEM, MEM/WB: 1,200 FFs), FPU pipeline stages (1,024 FFs), L1 cache tag/valid arrays and refill buffers (1,336 FFs), and 64-bit performance counters (640 FFs).

## 5. Verification Plans (≤ 6–8 lines total OR 3–4 bullets)

- •  Directed Unit Math & Edge-Case Testbenches: Exhaustively verify the Hardware Multiplier (DSP48E1 and Radix-4 Booth) and Divider across signed/unsigned operand spaces (MUL, MULH, MULHU, MULHSU, DIV, REM), edge cases (division by zero returning 0xFFFFFFFF, INT_MIN / -1 overflow), and IEEE 754 floating-point special values (±0, ±∞, and NaN propagation).

- •  Automated Bare-Metal C Benchmark Suite: Validate complete processor execution across arithmetic, array sorting, iterative logic, and pointer dereferencing using compiled bare-metal C programs (addition.c, fibonacci.c, sort.c, negative.c, xor.c) executed in cycle-accurate Vivado RTL simulation (xsim).

- •  Cache Latency & Pipeline Hazard Assertion Checks: Formally assert that L1 cache hits resolve in exactly 1 clock cycle while misses trigger 10-cycle simulated DRAM stall penalties; verify back-to-back load-use pipeline interlocks, consecutive RAW forwarding bypasses, and branch misprediction flushes under stress patterns.

- •  Hardware-in-the-Loop FPGA Validation: Deploy synthesizable bitstreams onto the Digilent Nexys A7 FPGA to verify real-time execution at 100 MHz and 1 Hz slow clock, reading 32-bit PC / Machine Code via the 8-digit seven-segment display, diagnosing hazards via 16 LEDs with pulse stretchers, and streaming CSR cache hit/miss statistics via UART.

## 6. Gantt Chart

Detailed parallel work schedule comprising 22 concrete, testable activities distributed across all five team members. Active work dates are marked in green shading. Key project milestones are highlighted: Milestone 1 Core Review on Oct 25, Minimum System Commitment on Nov 1, and Final FPGA Bitstream Verification on Nov 8.

|  |  | October 2026 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | November 2026 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Concrete Testable Activity | Owner | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1. 5-Stage base pipeline registers & datapath integration | Praneeth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Hazard Detection Unit & forwarding multiplexers RTL | Praneeth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. Static 32-entry Branch Target Buffer (BTB) RTL | Praneeth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. Automated C trace suite & full-system testbench | Praneeth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. Control Unit instruction decoder & stall logic | Rohith |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. Direct-mapped L1 Data Cache & write buffer FSM | Rohith |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7. Performance Counter CSRs (cycle, instret, misses) | Rohith |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8. Zbb Bit-Manipulation ALU (CLZ, CTZ, POPCNT) | Rohith |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9. Artix-7 DSP48E1 hardware multiplier instantiation | Siddharth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. Synthesizable Radix-4 Modified Booth Multiplier RTL | Siddharth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11. Dual-mode multiplier unit wrapper (sw[3] selection) | Siddharth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12. IEEE 754 single-precision FPU core (FADD, FMUL, FDIV) | Siddharth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13. IF & ID pipeline register logic & instruction fetch | Ramana |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14. Direct-mapped L1 Instruction Cache & 10-cycle FSM | Ramana |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15. Cache hit/miss testbenches & burst memory model | Ramana |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16. EX/MEM pipeline registers & store byte-enable masking | Sahithi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17. Multi-cycle Radix-2 Non-Restoring Divider RTL | Sahithi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18. Divider stall handshaking & divide-by-zero handling | Sahithi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19. Milestone 1 Core Review & Integration Check | ALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20. Minimum System Commitment Met (All 9 Features) | ALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21. Nexys A7 FPGA Demo: 7-Seg, LEDs, 1Hz Clk, UART streaming | ALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22. Final Timing Closure, Bitstream Generation & Grading Demo | ALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 7. Risks & Challenges (≤ 5 bullets, 1 line each)

- 1. Multi-Cycle Stall Interlocking: Coordinating concurrent stall requests from the divider (div_busy), FPU (fpu_busy), and L1 cache miss refill FSMs without causing pipeline deadlocks in the Hazard Detection Unit.

- 2. Timing Closure on Arithmetic Paths: Meeting 100 MHz setup timing constraints on cascading 32-bit DSP48E1 multiplier slices and FPU normalization datapaths on Artix-7 fabric.

- 3. Memory Coherency During Cache Refills: Ensuring instruction fetch and data memory accesses remain coherent without stale data hazards during 10-cycle burst BRAM refills.

- 4. IEEE 754 Corner Case Compliance: Accurately handling IEEE 754 boundary conditions including NaN quiet/signaling propagation, signed zeros (±0), overflow saturation, and round-to-nearest-even.

- 5. Speculative Branch Flush Latency: Ensuring 2-cycle branch penalty flushes correctly invalidate speculative instructions in IF and ID without corrupting architectural register state.

## 8. Response to TA feedback (for the project abstract)

Detailed technical responses addressing TA evaluation feedback regarding the base pipeline, hardware contributions, hazard handling, and verification methodology:

- 1. Limited hardware contribution in base 3-stage core: The project has expanded the hardware scope to a complete 5-stage pipelined processor featuring nine fully integrated hardware modules, including dual Artix-7 DSP48E1 and Radix-4 Booth multipliers, multi-cycle division, an IEEE 754 FPU, separate L1 instruction/data caches, a 32-entry BTB branch predictor, and passive performance counter CSRs.

- 2. Hazard resolution without software delays: All data hazards (distance-1 and distance-2 RAW) are resolved entirely in hardware using dual 3:1 operand forwarding multiplexers, and load-use hazards trigger automatic single-cycle hardware interlocks with NOP bubble injection without requiring compiler delays.

- 3. Demonstration of realistic workloads: Processor execution is verified using compiled bare-metal C benchmark programs (addition.c, fibonacci.c, sort.c, negative.c, xor.c) with cycle-accurate terminal profiling and CSR hardware performance counter verification rather than synthetic stimulus vectors.

- 4. Realistic timing diagram showing hazards: A full 24-cycle timing diagram has been included that explicitly details multi-cycle divider execution, single-cycle DSP multiplication, load-use stall freezes, and 2-cycle branch flushes, replacing any idealized hazard-free assumptions.

- 5. TA GitHub Repository Collaboration: Course TAs have been confirmed as collaborators with full read/write access to the official project GitHub repository: https://github.com/Rohith-Datta/RV32IM-5Stage-Core.git.