# Project Abstract (2-3 pages)

## Title

Pipelined RV32IMF RISC-V Core with L1 Cache Hierarchy, Hardware FPU, and IEEE 754 Floating-Point Support

Group Number: 13

## 1. System Description

Briefly explain (≤ 3 lines):

- •  What the system does: It is a custom 5-stage pipelined RISC-V (RV32IMF) processor featuring hazard detection and data forwarding, a dual-architecture hardware multiplier (supporting both native Artix-7 DSP48E1 slices and a Radix-4 Modified Booth Multiplier), direct-mapped L1 Instruction and Data Caches (simulating 10-cycle main memory vs. 1-cycle cache access to demonstrate cache impact), and an IEEE 754-compliant FPU supporting FADD.S, FSUB.S, FMUL.S, and FDIV.S.

- •  What problem it solves: It addresses the performance bottlenecks caused by software-emulated floating-point math and high memory-access latency. By simulating realistic main memory at 10 clock cycles and cache hits at 1 cycle, the design pedagogically demonstrates why caches are essential. Dedicated Artix-7 DSP48E1 blocks provide single-cycle integer multiplication without consuming general LUT fabric.

- •  High-level functionality: The system fetches instructions from a fast L1 I-Cache (1-cycle hit), with main memory modelled at 10 cycles to make cache misses tangible. Data accesses go through a direct-mapped L1 D-Cache with the same latency model. The 5-stage pipeline uses forwarding to eliminate data hazards, dedicated DSP48E1 slices for single-cycle MUL, and a dedicated IEEE 754 FPU for floating-point operations.

## 2. Use Cases

Give 2-3 examples (each ≤ 2 lines):

- •  Scientific and Engineering Computing: Used in embedded systems requiring precise floating-point arithmetic (e.g., sensor fusion, control algorithms). The FPU handles FADD.S/FMUL.S operations in hardware, eliminating costly software emulation.

- •  Digital Signal Processing (DSP): Used in audio or communications equipment for filtering and signal transformation. The hardware multiplier and FPU together accelerate convolution and FFT-style workloads.

- •  Embedded Control Systems: Used to run motor controllers or robotics algorithms that mix integer pipeline logic with floating-point PID computations, benefiting from the L1 I-Cache to reduce fetch latency.

## 3. FPGA Relevance

Give 2-3 points (each ≤ 2 lines):

- •  Custom Hardware Execution: FPGAs allow us to physically instantiate a dedicated IEEE 754 FPU and hardware multiplier that standard soft-core processors lack, enabling cycle-accurate testing of our exact design.

- •  Parallel Pipeline Stages: An FPGA runs all 5 pipeline stages — IF, ID, EX, MEM, WB — concurrently in real hardware, proving that forwarding and hazard detection work under real timing constraints.

- •  True Timing Verification: Software simulation cannot expose physical clock-domain issues. Running on an FPGA confirms that cache access, FPU latency, and pipeline stalls meet real-world timing.

## 4. System Scope

#### Minimum System (Commitment)

- 1. Implement a 5-stage pipeline (IF–ID–EX–MEM–WB) with integrated hazard detection and full data forwarding.

- 2. Implement a high-speed hardware multiplier supporting RV32M MUL/MULH/MULHU/MULHSU instructions using dedicated Artix-7 DSP48E1 slices, alongside a synthesizable Radix-4 Modified Booth Multiplier selectable via FPGA switch.

- 3. Build a direct-mapped L1 Instruction Cache (L1 I-Cache) with main memory simulated at 10 clock cycles and cache hits at 1 cycle, demonstrating the real-world need for caches.

- 4. Implement an IEEE 754-compliant FPU supporting FADD.S, FSUB.S, FMUL.S, and FDIV.S.

#### Goal System

- 5. Integrate a hardware divider (DIV/REM) with a dedicated pipeline stall controller.

- 6. Implement a direct-mapped L1 Data Cache (L1 D-Cache) with the same 10-cycle simulated main memory latency model, showcasing cache benefits for load/store operations.

- 7. Add branch prediction using a 32-entry Branch Target Buffer (BTB) to minimize speculative branch penalty.

- 8. Add passive performance counter CSRs to track cycle counts, cache hits, and cache misses (quantitatively demonstrating the cache speedup in a running program).

#### Stretch Goal

- 9. Implement the RISC-V Bit-Manipulation extension (Zbb) for single-cycle bitwise operations (CLZ, CTZ, POPCNT, ANDN, etc.).

## 5. Design Goals

Clearly measurable:

- •  Latency (cycles): 1 clock cycle for standard integer instructions, single-cycle MUL (DSP slices), and L1 cache hits. Floating-point latencies: 4 clock cycles for FMUL.S (DSP mantissa multiplication), 5 clock cycles for FADD.S and FSUB.S (standard logic FSM, no DSPs), and 15 clock cycles for FDIV.S (Newton-Raphson algorithm with DSP iterative multiplication and BRAM lookup). 32 clock cycles for integer DIV (shift-and-subtract state machine using standard logic, no DSPs). 10 clock cycles simulated for main memory access (cache miss penalty). 1-cycle load-use stall; 2-cycle branch-misprediction flush.

- •  Throughput: Peak 1.0 IPC during hazard-free integer execution. Target a stable clock frequency of ≥ 50 MHz on the FPGA board. Cache hit rate goal: ≥ 90% for sequential benchmark programs, yielding effective memory latency well below 10 cycles. Input sizes bounded by BRAM: up to 4 KB I-Cache, 4 KB D-Cache, sufficient for typical benchmarks (matrix multiply, FIR filter).

- •  Resource usage: LUTs ≤ 5,000 for core + hazard logic + standard logic FPU (FADD.S/FSUB.S) + divider. BRAM ≤ 16 tiles for L1 I-Cache, L1 D-Cache, FDIV.S initial guess lookup table, and main memory. DSP Slices ≤ 6 for the hardware multiplier (MUL) and FPU datapath (FMUL.S mantissa multiply and FDIV.S Newton-Raphson iterations, utilizing native Artix-7 DSP48E1 primitives).

- •  Correctness (edge cases): 100% pass rate on RV32IMF assembly and directed floating-point tests. FPU correctly handles IEEE 754 special cases: ±0, ±∞, and NaN propagation. Division by zero handled gracefully (result = 0xFFFFFFFF per RISC-V spec). Flawless back-to-back data forwarding verified under consecutive hazard sequences.

## 6. Implementation Approach

Describe (≤ 5 lines):

- •  New modules: Dedicated Verilog modules for pipeline registers (IF/ID, ID/EX, EX/MEM, MEM/WB), Hazard Detection & Forwarding Unit, direct-mapped L1 I-Cache and L1 D-Cache with a 10-cycle simulated main-memory latency model, dual-engine Hardware Multiplier (1-cycle native Artix-7 DSP48E1 slice instantiation and Radix-4 Booth logic), 32-cycle shift-and-subtract Hardware Divider (standard logic, no DSPs), IEEE 754 FPU (5-cycle FADD.S/FSUB.S via standard logic; 4-cycle FMUL.S via DSP; 15-cycle FDIV.S via Newton-Raphson using DSP iterative multiplication and BRAM lookup), BTB Branch Predictor, and passive CSR performance counters.

- •  Datapath/control changes: The existing 3-stage datapath will be restructured into 5 stages with forwarding multiplexers inserted between EX/MEM and MEM/WB. The multiplier unit supports dual modes selectable at runtime: single-cycle Artix-7 DSP48E1 primitives for zero LUT overhead, or Radix-4 Modified Booth logic for architectural comparison. The Control Unit will be extended to decode RV32F FPU instructions and manage multi-cycle stall signals for the divider, FPU, and cache-miss penalty cycles.

- •  Verification approach: (1) Module-level directed testbenches for each unit (pipeline, FPU, caches, DSP & Booth multipliers, divider, BTB). Cache testbenches explicitly verify 1-cycle hits and 10-cycle miss penalties. (2) Cycle-accurate Vivado RTL simulation of the full processor running RISC-V assembly programs with CSR performance counter verification. (3) Physical FPGA hardware demonstration on the Nexys A7 board featuring 8-digit seven-segment PC/instruction readout, LED hazard status flags, and UART result streaming.

## 7. Demonstration Plan

Clearly define (≤ 4 lines):

- •  Input: A pre-compiled bare-metal C/assembly program (e.g., floating-point vector math, FIR filter, or arithmetic stress test) loaded into FPGA block RAM (BRAM) instruction memory, accompanied by data arrays and runtime interactive switch controls (sw[0]–sw[3]).

- •  Processing: The custom RV32IMF core executes instructions across the 5-stage pipeline, fetching from the L1 I-Cache (1-cycle hit vs. 10-cycle simulated miss penalty), executing single-cycle DSP48E1 or Radix-4 Booth multiplications, performing IEEE 754 floating-point operations, handling load/store accesses via L1 D-Cache, and tracking performance with CSR counters.

- •  Output: Live, multi-peripheral visual verification on the Digilent Nexys A7 FPGA: (1) 8-Digit Seven-Segment Display showing the 32-bit Program Counter (sw[2]=0) or 32-bit Machine Instruction Code (sw[2]=1); (2) 16 Diagnostic LEDs with pulse stretchers indicating Load-Use Hazard Stall (LED[0]), Forwarding Active (LED[1]), Branch Flush (LED[2]), Writeback Commit (LED[3]), Multiplier Active (LED[4]), Multiplier Mode (LED[5]), and committed low-byte results (LED[15:8]); (3) Execution pacing via 1 Hz Slow Clock (sw[1]=1) or pushbutton single-stepping (sw[0]=1, btnc); and (4) UART transmission of computed results and CSR cache hit/miss statistics to a PC terminal.

## 8. High Level Timeline

### Requirements

- Must include at least 5–6 concrete milestones

- Milestones cannot be generic like “Continue implementation”, “Work on project”

- Each milestone must produce (deliverable):

  - (code + testbench) OR FPGA demo (graded)

- At least one core functionality/module of the minimum system should be implemented and tested by Oct 25.

- Ensure minimum system (commitment) is met by Nov 1. This will leave some room to handle last-minute surprises.

| Week | Milestone |
| --- | --- |
| 5 - 11 Oct | Milestone: 5-Stage Pipeline Base + Hazard & Forwarding Unit + L1 I-Cache (with 10-cycle memory model) Deliverable: IF–WB pipeline registers, forwarding mux RTL, hazard unit RTL, direct-mapped I-Cache RTL with simulated 10-cycle main memory + tb_pipeline_base.v, tb_hazard.v, tb_icache.v (verifying 1-cycle hit and 10-cycle miss penalty) |
| 12 - 18 Oct | Milestone: Dual-Architecture Hardware Multiplier (DSP48E1 + Radix-4 Booth) + IEEE 754 FPU (FADD.S, FSUB.S, FMUL.S, FDIV.S) Deliverable: Artix-7 DSP48E1 multiplier RTL, Radix-4 Modified Booth RTL, FPU RTL (all 4 ops) + tb_mul.v, tb_fpu.v; FPU & Multiplier integrated into pipeline with stall/forwarding support. Multiplier verified for MUL/MULH/MULHU/MULHSU on both DSP and Booth cores. |
| 19 - 25 Oct | Milestone: Hardware Divider + L1 D-Cache + Core Integration (Oct 25 Core Deadline Met) Deliverable: Divider RTL, D-Cache RTL + integrated tb_core.v proving integer/float/memory execution on FPGA demo |
| 26 Oct - 1 Nov | Milestone: Branch Prediction (32-entry BTB) + Performance Counter CSRs + Full System Integration (Nov 1 Commitment Met) Deliverable: BTB RTL, CSR counter RTL + complete Minimum & Goal System FPGA demo via 7-segment display, LEDs, and UART |
| 2 - 6 Nov | Milestone: Zbb Bit-Manipulation Extension + Buffer Week, Edge-Case Debugging & Final Documentation Deliverable: Zbb RTL (CLZ, CTZ, POPCNT), final project report, timing diagrams, code cleanup |

## 9. Team Work Plan

- All five members co-own the 5-stage pipeline base. Individual specialisations are listed below. Weekly sync meetings will review integration status, and a shared GitHub repository (feature-branch workflow) ensures continuous integration and code review.

- All members collaborate via GitHub pull requests with mandatory peer review before merging.

| Member | Responsibilities |
| --- | --- |
| Pisini Ramana | 5-Stage Pipeline (IF/ID stage lead) + L1 I-Cache design & verification + System Integration |
| Chimbili Sahithi | 5-Stage Pipeline (EX/MEM stage lead) + Hardware Divider (multi-cycle, stall controller) + Integration |
| Siddharth Misro | 5-Stage Pipeline (WB stage & forwarding lead) + DSP-Block Hardware Multiplier (Artix-7 DSP48E1 & Radix-4 Booth, MUL/MULH/MULHU/MULHSU) + IEEE 754 FPU (FADD.S, FSUB.S, FMUL.S, FDIV.S) + Integration |
| Bandaru Rohith Datta | 5-Stage Pipeline (Control Unit lead) + L1 D-Cache + Performance Counter CSRs + Zbb Extension + Integration |
| Sigilipelli Praneeth Sai Bharath | 5-Stage Pipeline (Hazard Detection Unit lead) + Static Branch Prediction (32-entry BTB) + Integration |

## 10. Github Link

- https://github.com/Rohith-Datta/RV32IM-5Stage-Core.git

-----------------------------------------------------------------------------------------------------------------------

Project Plan is Not accepted if:

- Timeline missing or milestones are generic

- No clear demo plan