# Block Diagram Report

# Pipelined RV32IMF RISC-V Core with L1 Cache Hierarchy, Hardware FPU, and IEEE 754 Floating-Point Support

Group Number: 13 | CS2202L System Design Lab

Target FPGA: Xilinx Artix-7 XC7A100T-1CSG324C (Digilent Nexys A7-100T Board)

Team Members & Specializations:

- •  Pisini Ramana: 5-Stage Pipeline (IF/ID stage lead) + L1 I-Cache design & verification + System Integration

- •  Chimbili Sahithi: 5-Stage Pipeline (EX/MEM stage lead) + Hardware Divider (multi-cycle, stall controller) + Integration

- •  Siddharth Misro: 5-Stage Pipeline (WB stage & forwarding lead) + Hardware Multiplier (Artix-7 DSP48E1 & Radix-4 Booth) + IEEE 754 FPU + Integration

- •  Bandaru Rohith Datta: 5-Stage Pipeline (Control Unit lead) + L1 D-Cache + Performance Counter CSRs + Zbb Extension + Integration

- •  Sigilipelli Praneeth Sai Bharath: 5-Stage Pipeline (Hazard Detection Unit lead) + Static Branch Prediction (32-entry BTB) + Integration

## 1. Top-Level Hardware Block Diagram (FULL PAGE)

### Table 1.1: Arithmetic Units Implementation Specification

| Instruction / Unit | Clock Cycles | Hardware Implementation Method |
| --- | --- | --- |
| MUL (Integer) | 1 | Direct combinational execution utilizing FPGA DSP Slices. |
| FMUL.S (Float) | 4 | Multi-cycle FSM utilizing DSP Slices for 24-bit mantissa multiplication. |
| FADD.S (Float) | 5 | Multi-cycle FSM using Standard Logic (LUTs, Adders, Shifters). No DSP slices. |
| FSUB.S (Float) | 5 | Multi-cycle FSM using Standard Logic (inverts sign bit, shares FADD hardware). No DSP slices. |
| FDIV.S (Float)* | 15 | Newton-Raphson Algorithm utilizing DSP Slices (for iterative multiplication) and BRAM (for initial guess lookup). |
| DIV (Integer) | 32 | Shift-and-Subtract state machine using Standard Logic (LUTs and Registers). No DSP slices. |

### Table 1.2: Architectural Signal & Bus Width Dictionary

| Signal Name | Source Module | Destination Module | Width | Function / Operational Description |
| --- | --- | --- | --- | --- |
| clk | Master Oscillator | All Pipeline Registers & Sequential Blocks | 1 bit | 50 MHz primary core clock (derived from 100 MHz board oscillator) distributed to all synchronous elements. |
| reset_n | Reset Synchronizer | All Pipeline Registers & Sequential Blocks | 1 bit | Active-low synchronous reset initializing PC and control state. |
| pc | PC Register | L1 I-Cache, Adder+4, Feature 7 BTB, IF/ID Reg | 32 bits | Current 32-bit instruction fetch address. |
| pc_plus_4 | Adder (+4) | PC Multiplexer (input 00), IF/ID Register | 32 bits | Sequential instruction link address (PC + 4). |
| btb_target | Feature 7 BTB | PC Multiplexer (input 01) | 32 bits | Speculative branch target address on valid BTB tag match. |
| btb_hit | Feature 7 BTB | Fetch Control Logic | 1 bit | Asserted when current PC matches a valid branch entry in the BTB. |
| instr | Feature 3 L1 I-Cache | IF/ID Pipeline Register | 32 bits | Fetched 32-bit RISC-V machine instruction word. |
| icache_stall | Feature 3 L1 I-Cache | Hazard Detection Unit | 1 bit | Stall hold requested during 10-cycle simulated main memory burst refill. |
| opcode | Instruction [6:0] | Control Unit, Hazard Detection Unit | 7 bits | RV32I/M base and extension opcode identifier. |
| funct3 | Instruction [14:12] | Control Unit, ALU Control, Branch Unit | 3 bits | Sub-operation selector for arithmetic, branches, loads, and stores. |
| funct7 | Instruction [31:25] | Control Unit, ALU Control, ID/EX Reg | 7 bits | Extended operation selector for math, FPU, and Zbb instructions. |
| rs1, rs2 | Instruction [19:15, 24:20] | Register File, Hazard Unit, ID/EX Reg | 5 bits | Source register indices 1 and 2 (x0 to x31). |
| rd | Instruction [11:7] | ID/EX -> EX/MEM -> MEM/WB RegFile Write | 5 bits | Destination register index retiring in Writeback stage. |
| imm | Immediate Generator | ID/EX Pipeline Register | 32 bits | Sign-extended 32-bit immediate (I, S, B, U, J format decoders). |
| pc_stall | Hazard Detection Unit | PC Register Enable (~pc_stall) | 1 bit | Pipeline freeze holding PC on load-use, cache, or math stalls. |
| if_id_stall | Hazard Detection Unit | IF/ID Register Enable (~if_id_stall) | 1 bit | Pipeline freeze holding IF/ID register on load-use hazard. |
| id_ex_flush | Hazard Detection Unit | ID/EX Pipeline Register Clear | 1 bit | Injects synchronous NOP bubble into EX stage on hazard or branch. |
| if_id_flush | Hazard Detection Unit | IF/ID Pipeline Register Clear | 1 bit | Annuls speculative instruction in IF/ID on resolved branch taken. |
| RegWrite | Control Unit | ID/EX -> EX/MEM -> MEM/WB -> RegFile we | 1 bit | Register write enable strobe committing wb_data into RegFile. |
| MemRead | Control Unit | ID/EX -> EX/MEM -> D-Cache, Hazard Unit | 1 bit | Memory read strobe asserted for load instructions (LB, LH, LW). |
| MemWrite | Control Unit | ID/EX -> EX/MEM -> L1 D-Cache | 1 bit | Memory write strobe asserted for store instructions (SB, SH, SW). |
| MemToReg | Control Unit | ID/EX -> EX/MEM -> MEM/WB -> WB Mux | 2 bits | Selects WB data source: 00:ALU/Math, 01:Memory, 10:PC+4, 11:CSR. |
| forward_a, b | Forwarding Unit | Forwarding Multiplexers A and B | 2 bits | Selects operand bypass: 00:RegFile, 01:EX/MEM stage, 10:MEM/WB stage. |
| mul_out | Feature 2 Multiplier | EX Result Multiplexer (input 001) | 32 bits | 1-cycle hardware multiplier output (MUL, MULH, MULHU, MULHSU via DSP slices). |
| div_out | Feature 5 Divider | EX Result Multiplexer (input 010) | 32 bits | 32-cycle shift-and-subtract divider quotient or remainder output. |
| div_busy | Feature 5 Divider | Hazard Detection Unit | 1 bit | Multi-cycle stall signal holding pipeline during 32 division cycles. |
| fpu_out | Feature 4 FPU | EX Result Multiplexer (input 011) | 32 bits | IEEE 754 float result: FMUL.S (4c DSP), FADD.S/FSUB.S (5c logic), FDIV.S (15c NR). |
| fpu_busy | Feature 4 FPU | Hazard Detection Unit | 1 bit | Multi-cycle stall handshake holding pipeline during 4/5/15-cycle FPU operations. |
| ex_result | EX Result Multiplexer | EX/MEM Register, Forwarding Distance-1 | 32 bits | Selected 32-bit execution result (ALU, multiplier, FPU, or address). |
| branch_taken | Branch Unit (EX) | PC Multiplexer, Hazard Unit, BTB | 1 bit | Asserted when branch condition is evaluated TRUE in EX stage. |
| dcache_stall | Feature 6 L1 D-Cache | Hazard Detection Unit | 1 bit | Stall hold requested during 10-cycle simulated D-Cache miss refill. |
| wb_data | Writeback Mux (4:1) | Register File wdata, Forwarding Distance-2 | 32 bits | Final committed 32-bit data written back into destination register. |

## 2. Timing Diagram (FULL PAGE)

Requirements Satisfied

- 29 Clock Cycles: The timing diagram covers 29 cycles, exceeding the minimum requirement of 20 cycles.

- L1 Data Cache Miss (Cycles 4–13): Instruction I1 (LW x5, 0(x10)) encounters a cache miss. The mem_stall signal remains active for 10 cycles while data is fetched from main memory.

- Data Forwarding (Cycle 14): The value x5 = 42 is forwarded to I2 (ADDI x6, x5, 4), which calculates x6 = 46.

- Branch Hazard (Cycles 15–16): Instruction I3 (BNE x6, x0, 0x20) uses x6 = 46 and takes the branch. The branch_flush signal invalidates I4, replacing it with a NOP bubble.

- FPU Execution (Cycles 18–26): The FPU remains busy for 9 continuous cycles while executing:

  - I5 (FADD.S f3, f1, f2): Takes 5 cycles using the standard logic FSM and calculates 1.5 + 2.5 = 4.0.

  - I6 (FMUL.S f5, f3, f4): Takes 4 cycles using the DSP-accelerated FSM and calculates 4.0 × 2.0 = 8.0, using forwarded f3.

- Memory Store (Cycles 18–28): Instruction I7 (SW x6, 0(x11)) remains in ID during FPU execution, enters EX in Cycle 27, and writes 46 to memory address 0x200 in Cycle 28.

- Register Writebacks:

  - Cycle 14: x5 = 42

  - Cycle 16: x6 = 46

  - Cycle 24: f3 = 4.0

  - Cycle 28: f5 = 8.0

- Realistic Pipeline Behavior: The diagram demonstrates data and control hazards, stalls, forwarding, branch flushing, multi-cycle FPU execution, and register and memory writes.

3. Module Summary (Table Only)

Complete structural breakdown of all 23 hardware modules comprising the extended RV32IMF processor core. Every module is implemented in synthesizable Verilog HDL and verified via cycle-accurate testbenches.

| Module | Purpose | File | Software/Hardware module? |
| --- | --- | --- | --- |
| pipeline_5stage | Top-level structural processor core integrating IF, ID, EX, MEM, WB | feature_1/modules/pipeline_5stage.v | Hardware (Verilog) |
| if_id_reg | IF/ID pipeline register with synchronous stall freeze and branch flush | feature_1/modules/if_id_reg.v | Hardware (Verilog) |
| decode_stage | Instruction decoder, immediate generator, 32-entry dual-port RegFile | feature_1/modules/decode_stage.v | Hardware (Verilog) |
| id_ex_reg | ID/EX pipeline register holding operands and control lines for EX | feature_1/modules/id_ex_reg.v | Hardware (Verilog) |
| execute_stage | Base 32-bit ALU, branch condition comparator, EX output multiplexer | feature_1/modules/execute_stage.v | Hardware (Verilog) |
| hazard_unit | Hazard Detection Unit: load-use stall freeze, branch flush, math stalls | feature_1/modules/hazard_unit.v | Hardware (Verilog) |
| forwarding_unit | Forwarding Unit: selects EX/MEM and MEM/WB bypass paths (Mux A/B) | feature_1/modules/hazard_unit.v | Hardware (Verilog) |
| ex_mem_reg | EX/MEM pipeline register passing memory address, write data, control | feature_1/modules/ex_mem_reg.v | Hardware (Verilog) |
| memory_stage | Memory interface with store byte-masking and load sign-extension | feature_1/modules/memory_stage.v | Hardware (Verilog) |
| mem_wb_reg | MEM/WB pipeline register passing load data and ALU result to WB | feature_1/modules/mem_wb_reg.v | Hardware (Verilog) |
| writeback_stage | Writeback multiplexer selecting memory, ALU, link PC, or CSR data | feature_1/modules/writeback_stage.v | Hardware (Verilog) |
| multiplier_unit | Feature 2: Dual-Mode Multiplier (1-cycle Artix-7 DSP48E1 & Radix-4 Booth) | feature_2/modules/multiplier_unit.v | Hardware (Verilog) |
| multiplier_dsp | Native Artix-7 DSP48E1 slice hardware multiplier: 1-cycle combinational execution | feature_2/modules/multiplier_dsp.v | Hardware (Verilog) |
| booth_radix4_multiplier | Synthesizable Radix-4 Modified Booth Multiplier RTL for architectural comparison | feature_2/modules/booth_radix4_multiplier.v | Hardware (Verilog) |
| icache_controller | Feature 3: Direct-mapped 2 KB L1 Instruction Cache (1-cyc hit, 10-cyc miss FSM) | feature_3/icache_controller.v | Hardware (Verilog) |
| fpu_single_core | Feature 4: IEEE 754 FPU (FMUL.S: 4c DSP; FADD.S/FSUB.S: 5c logic; FDIV.S: 15c NR with BRAM) | feature_4/fpu_single_core.v | Hardware (Verilog) |
| divider_unit | Feature 5: 32-cycle Shift-and-Subtract integer divider using Standard Logic (no DSPs) | feature_5/divider_unit.v | Hardware (Verilog) |
| dcache_controller | Feature 6: Direct-mapped 2 KB L1 Data Cache & write buffer | feature_6/dcache_controller.v | Hardware (Verilog) |
| btb_branch_pred | Feature 7: Static 32-entry Branch Target Buffer (BTB) predictor | feature_7/btb_branch_pred.v | Hardware (Verilog) |
| csr_perf_counters | Feature 8: Passive performance CSR monitoring registers (CYCLE, INSTRET, misses) | feature_8/csr_perf_counters.v | Hardware (Verilog) |
| bitmanip_unit | Feature 9: Zbb bit manipulation ALU (CLZ, CTZ, POPCNT, MIN, MAX) | feature_9/bitmanip_unit.v | Hardware (Verilog) |
| fpga_top_feature1 | Dedicated synthesizable FPGA top wrapper for Feature 1 (7-seg & LEDs) | feature_1/modules/fpga_top_feature1.v | Hardware (Verilog) |
| fpga_top_feature2 | Dedicated synthesizable FPGA top wrapper with DSP/Booth runtime toggle | feature_2/modules/fpga_top_feature2.v | Hardware (Verilog) |

## 4. Hardware Resource Estimates

Resource utilization estimates for the extended RV32IMF processor core targeted to the Xilinx Artix-7 XC7A100T-1CSG324C FPGA (Digilent Nexys A7-100T board). The design fits comfortably within the target device budget while keeping total LUT consumption well under the 5,000 LUT commitment.

| Resource | Maximum Available | Estimated Usage | % utilization |
| --- | --- | --- | --- |
| BRAM | 135 tiles (4,860 Kb) | 8 tiles | 5.9 % |
| DSP blocks | 240 slices (DSP48E1) | 4 slices | 1.7 % |
| LUTs | 63,400 | 4,850 | 7.6 % |
| FF | 126,800 | 4,200 | 3.3 % |

Detailed Resource Breakdown & Engineering Justifications:

- •  BRAM (8 tiles / 5.9%): Allocated across the 2 KB L1 I-Cache data array (4 tiles), 2 KB L1 D-Cache data array (3 tiles), and the FDIV.S Newton-Raphson initial guess reciprocal lookup table ROM (1 tile). Main memory is simulated in BRAM block storage.

- •  DSP Blocks (4 slices / 1.7%): Dedicated native Artix-7 DSP48E1 slices handle single-cycle 32-bit integer multiplication (MUL, MULH), FMUL.S 24-bit mantissa multiplication (4-cycle FSM), and FDIV.S Newton-Raphson iterative multiplier stages. In strict adherence to specifications, NO DSP slices are used for FADD.S, FSUB.S, or integer DIV.

- •  LUTs (4,850 LUTs / 7.6%): Standard logic (LUTs, Adders, Shifters) synthesizes the 5-cycle FADD.S/FSUB.S floating-point units and the 32-cycle Shift-and-Subtract integer DIV state machine, alongside the 5-stage core logic (2,100 LUTs), Radix-4 Booth logic, CSRs, and caches (4,850 LUTs total, comfortably under ≤ 5,000 LUTs).

- •  Flip-Flops (4,200 FFs / 3.3%): Pipeline registers (IF/ID, ID/EX, EX/MEM, MEM/WB: 1,200 FFs), FPU pipeline stages (1,024 FFs), L1 cache tag/valid arrays and refill buffers (1,336 FFs), and 64-bit performance counters (640 FFs).

## 5. Verification Plans (≤ 6–8 lines total OR 3–4 bullets)

1. Directed Unit Math & Edge-Case Testing: Test all arithmetic units for correct results and execution time: integer multiplication (1 cycle), division (32 cycles), floating-point multiplication (4 cycles), addition/subtraction (5 cycles), and division (15 cycles). Check special cases like division by zero, overflow, ±0, infinity, and NaN.

2. Automated C Benchmark Testing: Run bare-metal C programs for addition, Fibonacci, sorting, negative numbers, XOR, and pointer operations in Vivado simulation to verify correct processor execution.

3. Cache & Pipeline Hazard Testing: Verify that cache hits take 1 cycle and cache misses cause a 10-cycle memory stall. Test load-use hazards, data forwarding, and flushing incorrect instructions after branch mispredictions.

4. FPGA Hardware Testing: Run the processor on the Digilent Nexys A7 FPGA at 50 MHz core clock and a 1 Hz slow clock. Display the program counter and machine code on the seven-segment display, use LEDs to monitor hazards, and send cache hit/miss statistics through UART.

## 6. Gantt Chart

Detailed parallel work schedule comprising 24 concrete, testable activities distributed across all five team members and collaborative ALL integration windows. Active engineering dates are marked in green shading (■). Critical project milestones are prominently highlighted in amber/gold shading with distinct markers (◆ / ★): Milestone 1 Core Review on Oct 25, Minimum System Commitment on Nov 1, and Final FPGA Bitstream Verification on Nov 8. Mid-pipeline subsystem integration (Oct 11–17) and pre-commitment datapath testing (Oct 26–31) are actively scheduled under the ALL section so that team members collaborate continuously on datapath interlocks and hazard verification without idle periods.

|  |  | October 2026 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | November 2026 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Concrete Testable Activity | Owner | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25★ | 26 | 27 | 28 | 29 | 30 | 31 | 1★ | 2 | 3 | 4 | 5 | 6 | 7 | 8★ |
| 1. IF & ID pipeline register logic & instruction fetch | Ramana |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Direct-mapped L1 Instruction Cache & 10-cycle FSM | Ramana |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. Cache hit/miss testbenches & burst memory model | Ramana |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. EX/MEM pipeline registers & store byte-enable masking | Sahithi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. Multi-cycle Radix-2 Non-Restoring Divider RTL | Sahithi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. Divider stall handshaking & divide-by-zero handling | Sahithi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7. Artix-7 DSP48E1 hardware multiplier instantiation | Siddharth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8. Synthesizable Radix-4 Modified Booth Multiplier RTL | Siddharth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9. Dual-mode multiplier unit wrapper (sw[3] selection) | Siddharth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. IEEE 754 single-precision FPU core (FADD, FMUL, FDIV) | Siddharth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11. Control Unit instruction decoder & stall logic | Rohith |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12. Direct-mapped L1 Data Cache & write buffer FSM | Rohith |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13. Performance Counter CSRs (cycle, instret, misses) | Rohith |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14. Zbb Bit-Manipulation ALU (CLZ, CTZ, POPCNT) | Rohith |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15. 5-Stage base pipeline registers & datapath integration | Praneeth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16. Hazard Detection Unit & forwarding multiplexers RTL | Praneeth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17. Static 32-entry Branch Target Buffer (BTB) RTL | Praneeth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18. Automated C trace suite & full-system testbench | Praneeth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19. Pipeline Subsystem Integration & Stall Interlocking | ALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20. ★ MILESTONE 1: Core Subsystem Review & Integration Check | ALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ◆ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21. Full-System 9-Feature Datapath Integration & Interconnect | ALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22. ★ MILESTONE 2: Minimum System Commitment Met (All 9 Features) | ALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ◆ |  |  |  |  |  |  |  |
| 23. Nexys A7 FPGA Demo: 7-Seg, LEDs, 1Hz Clk, UART streaming | ALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24. ★ FINAL MILESTONE: Timing Closure & Project Grading Demo | ALL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ★ |

Gantt Chart Legend & Milestone Highlights:

•  Active Engineering Work (■ Green Shading):Scheduled RTL design, logic synthesis, unit testbench verification, and FPGA prototyping.

•  Mid-Project Subsystem Integration (Oct 11–17 under ALL):Collaborative whole-team window where all members (especially pipeline and hazard leads) integrate incoming math units (DSP48E1 / Booth Multipliers) and L1 I-Cache into the 5-stage datapath with multi-cycle stall interlocking.

•  ★ Milestone 1 Core Review (Oct 25, Day 25 marked ◆):Core functionality milestone satisfied. 5-Stage pipeline, Hazard Detection Unit, operand forwarding, dual-mode multiplier, Radix-2 divider base, and L1 I-Cache fully functional and verified.

•  ★ Minimum System Commitment Met (Nov 01, Day 32 marked ◆):All 9 required project features (32-entry BTB, DSP/Booth Multipliers, Radix-2 Divider, IEEE 754 FPU, L1 I-Cache, L1 D-Cache, CSR counters, Zbb ALU, and Hazard Unit) fully integrated into core.

•  ★ Final Demonstration & Sign-Off (Nov 08, Day 39 marked ★):Full-speed 50 MHz timing closure achieved on Artix-7 FPGA fabric, bitstream generated, UART / 7-segment / LED diagnostics active, and final TA grading demonstration.

## 7. Risks & Challenges (≤ 5 bullets, 1 line each)

- Multi-Cycle Stall Handling: Managing stalls caused by the 32-cycle divider, 4/5/15-cycle FPU operations, and 10-cycle cache misses without causing deadlocks.

- Arithmetic Timing: Ensuring that 32-bit multiplication and floating-point division operations meet the 50 MHz timing requirement on the Artix-7 FPGA.

- Memory Consistency During Cache Refills: Ensuring that instruction and data memory accesses return correct and up-to-date values during 10-cycle cache refills.

- IEEE 754 Special Cases: Correctly handling floating-point special cases such as NaN, positive and negative zero, overflow, and convergence during Newton-Raphson division.

- Branch Flush Handling: Correctly flushing incorrect instructions from the IF and ID stages during a branch, with a 2-cycle penalty, without modifying the processor's actual register values incorrectly.

## 8. Response to TA feedback (for the project abstract)

1. Limits on Input Sizes:
To address the concern regarding the limits on input sizes due to limited hardware resources, we are setting a strict limit on our total Main Memory to 64 KB. This ensures that our entire memory easily fits inside the Artix-7 FPGA's built-in block memory (BRAM) without using up too many hardware resources. Consequently, the size of our inputs—such as the test arrays and matrices used during our demo—will be strictly limited so that they, along with the program code, fit entirely within this 64 KB space. Additionally, to match this memory size and properly demonstrate cache misses, our L1 Caches will be limited to 2 KB each

2.Cache Requirement:

Although memory access currently takes 1 CC in our design, real memory access takes multiple cycles. Our goal isn’t to optimise the existing FPGA processor, but to understand cache implementation and demonstrate its importance. So, we’ll simulate normal memory access taking 10 CCs and cache access taking 1 CC, showcasing the need for cache and making the FPGA simulation more realistic. So we are thinking not to remove cache.

3.  Feature modification:

We have removed MAC Coprocessor and replaced it with IEEE754 FPU.

Also we have moved Performance counters from Stretch to Goal section.

So hope project scope is fine now.