# Project Design Status Report

Title: Design and FPGA Implementation of a Pipelined RV32IMF Processor with 2KB L1 Caches, Dual Multipliers, 32-Cycle Newton-Raphson Divider, IEEE 754 FPU, Static BTB, and CSR Performance Counters

Group Number: Group 13

## 1. System Summary (≤ 4 lines)

- What the system does: A high-performance 5-stage pipelined RV32IMF processor integrating dual hardware multipliers (DSP48E1 single-cycle & Booth multi-cycle), a 32-cycle Newton-Raphson divider, an IEEE 754 single-precision FPU, 2KB direct-mapped L1 I/D-caches with 10-cycle refill FSMs and bypass mode, static BTB branch prediction, CSR performance counters, and real-time UART telemetry.

- Current stage: Implemented and fully verified on Digilent Basys 3 FPGA (Xilinx Artix-7 xc7a35tcpg236-1) at 50 MHz.

- Current working capability: Full RV32I integer set, RV32M mul/div (32 ccs), IEEE 754 float math, and 2KB L1 caches operational on hardware with live UART telemetry; Static BTB and CSR performance counters verified in simulation.

## 2. Scope Tracking

| Component | Description | Status (Done / Partial / Not Started) | Expected Completion Date | Evidence |
| --- | --- | --- | --- | --- |
| Minimum System | Feature 1: 5-Stage RV32I Pipelined Core with Full Forwarding & Hazard Unit | Done | 11 Oct | FPGA (Basys 3, UART telemetry verified) |
|  | Feature 2: RV32M Dual Multiplier (Single-Cycle DSP48E1 & 32-Cycle Booth) | Done | 17 Oct | FPGA (Basys 3, UART telemetry verified) |
|  | Feature 3: RV32M Hardware Divider (32-Cycle Newton-Raphson Method) | Done | 25 Oct | FPGA (Basys 3, UART telemetry verified) |
| Goal System | Feature 4: IEEE 754 Single-Precision Floating Point Unit (FADD/FSUB/FMUL/FDIV) | Done | 24 Oct | FPGA (Basys 3, UART telemetry verified) |
|  | Feature 6: Direct-Mapped 2KB L1 Instruction Cache (10-Cycle Stall Refill FSM) | Done | 23 Oct | FPGA (Basys 3, UART telemetry verified) |
|  | Feature 5: Direct-Mapped 2KB L1 Data Cache (Write-Through & Hardware Bypass Mode) | Done | 24 Oct | FPGA (Basys 3, UART telemetry verified) |
|  | Static Branch Prediction with Branch Target Buffer (Static BTB) | Partial | 30 Oct | Testbench (tb_btb simulation passed) |
|  | Hardware Performance Counters (CSR Cycle, Time, and Instret Counters) | Partial | 1 Nov | Testbench (tb_csr simulation passed) |
| Stretch Goal | RISC-V Zbb Bit-Manipulation Extension (CLZ, CTZ, CPOP, MIN/MAX) | Not Started | 5 Nov | Design: ALU decoder spec drafted |

Note:

- “Done” = working on FPGA; otherwise mark Partial

- Evidence: validation / FPGA result / Design: GIT commit reference

## 3. Design Goals Tracking

- Goals are measurable (see project abstract report)

| Metric | Target | Current | Status (Met / Close / Not Met) |
| --- | --- | --- | --- |
| Latency | Base: 1 cyc; Mul: 1 cyc; Div: ≤ 34 ccs; Cache miss: ≤ 10 cyc | Base: 1 cyc; Mul: 1 cyc; Div: 32 ccs (Newton-Raphson Method); Cache miss: 10 cyc (9 st+1 act) | Met |
| Throughput | Freq ≥ 50 MHz (Period ≤ 20.0 ns); Peak CPI ≈ 1.0 | 50.0 MHz on Basys 3 FPGA (WNS = +4.084 ns slack); CPI = 1.0 - 1.15 | Met |
| Resource Usage: LUT | < 20,800 LUTs (< 100% Basys 3 XC7A35T capacity) | ~7,850 LUTs (~37.7% utilization) | Met |
| Resource Usage: FF | < 41,600 Flip-Flops (< 100% Basys 3 capacity) | ~5,200 FFs (~12.5% utilization) | Met |
| Resource Usage: BRAM | ≤ 50 Block RAMs (36Kb blocks, Basys 3 limit) | 4.5 BRAM tiles (9.0%: 2KB I$, 2KB D$, unified memory) | Met |
| Resource Usage: DSP | ≤ 90 DSP48E1 slices (Basys 3 limit) | 4 DSP slices (4.4% utilization for fast multiplier) | Met |
| Correctness | 100% pass rate on cross-compiled bare-metal C test suite | 100% passed (Base integer, RV32M mul/div, FPU, 2KB L1 caches) | Met |

## 4. Demonstration Plan Status

- For 27 Oct (Mini Demo)

| Component | Planned | Current Status (1-2 lines) |
| --- | --- | --- |
| Input | Bare-metal C test programs compiled via GCC into hex .mem files, covering RV32I ALU, RV32M multiplication, 32-cycle Newton-Raphson division, and 2KB L1 cache transactions. | Fully prepared; automated test pipeline (run_c_tests.py) builds and initializes c_tests/ (division.c, mul.c, arithmetic.c) into instruction memory. |
| Processing | Real-time execution at 50 MHz on Basys 3 FPGA; 5-stage pipeline with hazard forwarding, 32-cycle Newton-Raphson divider stalls, and 10-cycle cache refill latency emulation. | Working on FPGA with clean timing closure (WNS = +4.084 ns); divider (32 ccs), dual-multiplier, and 2KB cache refill FSMs hardware-verified. |
| Output | 115200 baud UART telemetry streaming cycle-accurate execution logs, stall breakdowns, cache hits/misses, and test pass signature a0=16. | Fully functional; PowerShell scripts (read_uart.ps1) capture cycle-by-cycle telemetry, displaying 1-based cycles, stalls, and register verification. |

- For 7-8 Nov (Final Demo)

| Component | Planned | Current Status (1-2 lines) |
| --- | --- | --- |
| Input | Comprehensive computational benchmarks (CoreMark, Matrix Multiplication, FIR filtering) with dynamic UART program binary loading. | Benchmark test algorithms developed and compiled; UART bootloader module currently in RTL design. |
| Processing | Full RV32IMF core with active 2KB L1 I/D-caches, Static BTB branch prediction, IEEE 754 FPU math, and hardware CSR performance counters. | RV32IMF, FPU, and 2KB caches verified on FPGA; Static BTB branch predictor and CSR counters in integration testing. |
| Output | UART performance dashboard streaming IPC, total cycles, branch hit rate, cache hit rate, cache speedup vs. bypass mode, and test status. | Cache bypass mode telemetry verified; integrating CSR counter packet formatting for live benchmark metrics. |

## 5. Gantt Chart

- Implement and be ready with at least one core functionality by 25 Oct.

- Complete the full minimum system by 1 Nov, leaving buffer for last-minute surprises.

- Up to 25 Oct: update based on actual execution

- For after 25 Oct: fill as planned schedule

- Project Execution & Planning Schedule:

|  |  | Oct |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Nov |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Owner | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| RV32I 5-Stage Core Pipeline | All |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pipeline Hazard & Forwarding Unit | S. P. S. Bharath |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RV32M Multiplier (DSP & Booth) | S. P. S. Bharath |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2KB L1 Instruction Cache & Refill FSM | C. Sahithi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IEEE 754 FPU (FADD/FSUB/FMUL) | S. Misro |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2KB L1 Data Cache & Bypass Controller | B. R. Datta |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IEEE 754 FPU Divider & Exceptions | P. Ramana |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RV32M Divider (Newton Raphson, 32 ccs) | S. P. S. Bharath |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Static BTB Branch Prediction Unit | B. R. Datta |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSR Hardware Performance Counters | C. Sahithi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full System Integration & FPGA Verification | S. P. S. Bharath & Team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6. Risk Tracking

| Risk | Status (New / Ongoing / Resolved) | Impact (High, Medium) | Owner | Mitigation Plan (1 line) |
| --- | --- | --- | --- | --- |
| Multi-cycle Divider stall & pipeline hazard collision | Resolved | High | Sigilipelli Praneeth Sai Bharath | Implemented 32-cycle Newton-Raphson Method asserting div_busy to hold IF/ID/EX stages until quotient/remainder converge. |
| 2KB L1 Data Cache slow memory refill latency and stall overhead | Resolved | High | Bandaru Rohith Datta | Optimized 2KB D-Cache 10-cycle burst refill FSM (9 stall cycles) and built hardware Cache Bypass Mode switch. |
| Timing closure degradation at 50 MHz due to IEEE 754 FPU division | Resolved | Medium | Siddharth Misro & Pisini Ramana | Pipelined FADD/FMUL into 2 cycles and structured FDIV as iterative multi-cycle logic, achieving positive slack (WNS = +4.084 ns). |
| Pipeline deadlock under simultaneous cache miss and math stalls | Resolved | High | Sigilipelli Praneeth Sai Bharath | Centralized hazard detection unit with strict priority encoding (D-Cache > I-Cache > FPU > Divider). |
| 2KB L1 Instruction Cache miss latency overhead on tight loops | Resolved | Medium | Chimbili Sahithi | Direct-mapped 2KB tag matching with 10-cycle burst refill buffer to service loop execution with zero wait-states on hit. |
| Static BTB branch target address misprediction penalty | Ongoing | Medium | Bandaru Rohith Datta | Static branch target buffer with single-cycle decode-time redirection and 2-cycle flush recovery on misprediction. |

7. Per-Person Contribution

| Member | Planned | Status (Done, WIP, Not started) | Estimated Completion Date |
| --- | --- | --- | --- |
| Sigilipelli Praneeth Sai Bharath | RV32I 5-stage core pipeline design & execution datapath (Joint with All) | Done | 11 Oct |
|  | Centralized hazard detection, load-use interlocks & data forwarding unit | Done | 12 Oct |
|  | RV32M Hardware Multipliers (single-cycle DSP48E1 & 32-cycle Booth) | Done | 17 Oct |
|  | RV32M Hardware Divider (32-Cycle Newton-Raphson Method) & stall handshaking | Done | 25 Oct |
|  | System integration, automated C test harness & Vivado bitstream flow | Done | 25 Oct |
| Chimbili Sahithi | RV32I 5-stage core pipeline datapath development (Joint with All) | Done | 11 Oct |
|  | 2KB L1 Instruction Cache controller design & direct-mapped tag comparison | Done | 20 Oct |
|  | 2KB L1 Instruction Cache 10-cycle memory stall refill FSM implementation | Done | 23 Oct |
|  | CSR Hardware Performance Counters (Cycle, Time, Instret) architecture | WIP | 1 Nov |
| Bandaru Rohith Datta | RV32I 5-stage core pipeline control logic (Joint with All) | Done | 11 Oct |
|  | 2KB L1 Data Cache controller, direct-mapped cache memory & write-through logic | Done | 21 Oct |
|  | 2KB L1 Data Cache 10-cycle memory refill FSM & hardware cache bypass mode | Done | 24 Oct |
|  | Static BTB branch prediction unit & UART telemetry integration | WIP | 30 Oct |
| Siddharth Misro | RV32I 5-stage core pipeline ALU & register file (Joint with All) | Done | 11 Oct |
|  | IEEE 754 single-precision FPU core architecture & FADD / FSUB units | Done | 22 Oct |
|  | IEEE 754 single-precision Floating-Point Multiplier (FMUL) unit | Done | 24 Oct |
|  | Bare-metal C floating-point testbenches & algorithmic verification | Done | 25 Oct |
| Pisini Ramana | RV32I 5-stage core pipeline memory & writeback stages (Joint with All) | Done | 11 Oct |
|  | IEEE 754 single-precision Floating-Point Divider (FDIV) logic | Done | 23 Oct |
|  | IEEE 754 floating-point exception handling, rounding modes & status flags | Done | 24 Oct |
|  | Memory & FPU stress benchmark suites, hardware speedup profiling | Done | 25 Oct |

- For activities marked Done, mention the actual completion date.

- For others, provide the estimated completion date.

- WIP = Work in Progress.

## 8. GitHub Snapshot

- Repo link: https://github.com/praneethsaibharath/RISCV.git

| Member | Commits (count) |
| --- | --- |
| Sigilipelli Praneeth Sai Bharath | 90 |
| Siddharth Misro | 9 |
| Bandaru Rohith Datta | 8 |
| Chimbili Sahithi | 7 |
| Pisini Ramana | 7 |
| Total | 121 |

Key Commits (with 2-3 line description)

| Commit Title | Description |
| --- | --- |
| feat(core): implement 5-stage pipelined RV32I core with hazard forwarding | Built IF, ID, EX, MEM, and WB stages with full data forwarding and load-use hazard interlocks. Verified on Digilent Basys 3 FPGA via UART telemetry streaming program execution and register state. |
| feat(math): integrate RV32M dual-multiplier and 32-cycle Newton-Raphson divider | Added single-cycle DSP48E1 and 32-cycle Booth multiplier options, alongside an iterative 32-cycle Newton-Raphson Method divider. Implemented multi-cycle stall arbitration to freeze pipeline stages until completion. |
| feat(cache): add 2KB L1 I-cache and D-cache with 10-cycle refill FSM and bypass mode | Implemented 2KB direct-mapped instruction and data caches with write-through policy, tag validation, and 10-cycle memory latency stall emulation. Added hardware switchable cache bypass mode. |
| feat(fpu): implement IEEE 754 single-precision FPU and automated C test harness | Integrated pipelined FADD, FSUB, FMUL, and multi-cycle FDIV with IEEE 754 round-to-nearest-even and exception detection. Created automated GCC compiler flow and hex memory generation scripts. |

## 9. Changes Based on TA Inputs

- Feedback received in project presentation: TA highlighted the need for hardware stall observability, multi-cycle stall collision deadlock prevention across math units/caches, and realistic memory latency penalties.

- Changes implemented: Added real-time UART stall telemetry, prioritized the pipeline hazard detection logic, and engineered a 10-cycle memory refill FSM with hardware cache bypass mode.

| Feedback/Input | Original | Current | Change |
| --- | --- | --- | --- |
| Real-Time Hardware Observability of Stalls & Cache Refills | Internal performance counters and stall states were only observable via post-synthesis Vivado simulation waveforms. | Real-time 115200 baud UART telemetry streaming exact cycle counts, stall causes, memory refill durations, and pass/fail signatures. | Designed dedicated hardware UART packet formatting engine and interactive PowerShell monitor script (read_uart.ps1). |
| Multi-Cycle Hazard Deadlock Avoidance Between Execution Units | Distributed, unprioritized OR-gate stall requests feeding pipeline registers, risking dropped instructions under concurrent stalls. | Centralized hazard detection unit with strict priority encoding (D-Cache Refill > I-Cache Refill > FPU > Divider). | Re-architected hazard control logic to hold program counter and pipeline stages atomically without instruction duplication. |
| Realistic Memory Subsystem Latency Modeling | Single-cycle synchronous Block RAM memory responding immediately on clock edge without simulating external memory delay. | Memory controller with realistic 10-cycle penalty (9 stall cycles + 1 active cycle) and switchable cache bypass mode. | Implemented 10-cycle memory burst refill FSM and cache bypass mode to realistically demonstrate cache performance gains on FPGA. |